Manipulator Shape Tracing in MATLAB and Simulink

This example uses:

- Robotics System Toolbox

- Simulink

This example shows how to trace a predefined 3-D shape in space. Following a smooth, distinct path is useful in many robotics applications such as welding, manufacturing, or inspection. A 3-D trajectory is solved in the task space for tracing the MATLAB® membrane and is executed using the Sawyer robot from Rethink Robotics®. The goal is to generate a smooth path for the end effector of the robot to follow based on a set of waypoints.

The Trajectory Control Modeling with Inverse Kinematics example shows how to generate a closely discretized set of segments that can then be passed to an inverse kinematics solver to be solved using an iterative solution. However, this example offers an alternate approach to reduce computational complexity. This example splits path segments into just a few discrete points and uses smoothing functions to interpolate between the waypoints. This approach should generate a smoother trajectory and improve run-time efficiency.

Load the Robot

This example uses the Sawyer robot from Rethink Robotics®. Import the URDF file that specifies the rigid body dynamics. Set the DataFormat to use column vectors to define robot configurations. Simulink® uses column vectors. The task space limits are defined based on empirical data.

sawyer = loadrobot("rethinkSawyer",DataFormat="column");

taskSpaceLimits = [0.5 0.75; -0.125 0.125; 0.25 0.5];

numJoints = 8; % Number of joints in robot

Generate Set of Task-Space Waypoints

For this example, the goal is to get a set of path segments that trace the MATLAB® membrane logo. The membrane surface and the path segments are generated as cell arrays using the helper function generateMembranePaths. To visualize the paths overlaid on the surface, plot the surface using surf and the path segments by iterating through the path segment cell array. You can increase numSamples to sample more finely across the surface.

numSamples = 7;

[pathSegments,surface] = generateMembranePaths(numSamples,taskSpaceLimits);

% Visualize the output

figure

surf(surface{:},FaceAlpha=0.3,EdgeColor="none");

title("Path Segments Plotted on L-Membrane")

hold on

for i=1:numel(pathSegments)

segment = pathSegments{i};

plot3(segment(:,1),segment(:,2),segment(:,3),"x-",LineWidth=2);

end

hold off

To ensure that the robot can trace the output, visualize the shape in the robot workspace. Show the sawyer robot and plot the line segments in the same figure.

figure

show(sawyer);

hold on

title("Path Segments Plotted in Robot Workspace")

for i=1:numel(pathSegments)

segment = pathSegments{i};

plot3(segment(:,1),segment(:,2),segment(:,3),"x-",LineWidth=2);

end

view(135,20)

axis([-0.25 1 -.5 .5 -0.1 .75])

hold off

Create an Inverse Kinematics Solver

Create an inverse kinematics (IK) using the loaded sawyer rigid body tree. It is initially configured with a uniform set of weights, using the home configuration as the initial guess. Set the initial guess to the home configuration and the pose tolerances with uniform weights. The end effector for IK solver is the "right_hand" body of the robot.

s=rng(0); % Capture the current seed and seed the rng to 0.

ik = inverseKinematics(RigidBodyTree=sawyer);

ik.SolverParameters.AllowRandomRestarts = false;

initialGuess = sawyer.homeConfiguration;

weights = [1 1 1 1 1 1];

eeName = "right_hand";

Convert Task-Space Waypoints to Joint-Space Using Inverse Kinematics

Use the inverse kinematics solver to generate a set of joint space waypoints, which give the joint configurations for the robot at each point of the generated pathSegments. Each joint-space segment is filed into a matrix, jointPathSegmentMatrix, which is passed to the Simulink model as an input.

% Initialize the output matrix

jointPathSegmentMatrix = zeros(length(pathSegments),numJoints,numSamples);

% Define the orientation so that the end effector is oriented down

sawyerOrientation = axang2rotm([0 1 0 pi]);

% Compute IK at each waypoint along each segment

for i = 1:length(pathSegments)

currentTaskSpaceSegment = pathSegments{i};

currentJointSegment = zeros(numJoints, length(currentTaskSpaceSegment));

for j = 1:length(currentTaskSpaceSegment)

pose = [sawyerOrientation currentTaskSpaceSegment(j,:)'; 0 0 0 1];

currentJointSegment(:,j) = ik(eeName,pose,weights,initialGuess);

initialGuess = currentJointSegment(:,j);

end

jointPathSegmentMatrix(i,:,:) = (currentJointSegment);

end

Load Simulink Model

Use the shapeTracingSawyer model to execute the trajectories and simulate them on a kinematic model of the robot.

open_system("shapeTracingSawyer.slx")

The Simulink model has two main parts:

- The Trajectory Generation section takes the matrix of joint-space path segments, jointPathSegmentMatrix, and converts the segments to a set of discretized joint-space waypoints (joint configurations) at each time step in the simulation using a MATLAB function block. The Polynomial Trajectory Block converts the set of joint configurations to a smoothed joint space B-spline trajectory in time.

- The Robot Kinematics Simulation section accepts the joint-space waypoints from the smoothed trajectory and computes the resulting end-effector position for the robot.

Trajectory Generation

Robot Kinematics Simulation

Execute Joint-Space Trajectories in Simulink

Simulate the model to execute the generate trajectories.

sim("shapeTracingSawyer.slx")

View Trajectory Generation Results

The model outputs the robot joint configurations and the end-effector positions along each smoothed path trajectory. To work easily with MATLAB plotting tools, reshape the data.

% End effector positions

xPositionsEE = reshape(eePosData.Data(1,:,:),1,size(eePosData.Data,3));

yPositionsEE = reshape(eePosData.Data(2,:,:),1,size(eePosData.Data,3));

zPositionsEE = reshape(eePosData.Data(3,:,:),1,size(eePosData.Data,3));

% Extract joint-space results

jointConfigurationData = reshape(jointPosData.Data,numJoints,size(eePosData.Data,3));

Plot the new end-effector positions on the original membrane surface.

figure

surf(surface{:},FaceAlpha=0.3,EdgeColor="none");

hold on

plot3(xPositionsEE,yPositionsEE,zPositionsEE)

grid on

title(["Smoothed End-Effector Trajectories","Plotted on L-Membrane Surface"])

hold off

Visualize Robot Motion

In addition to the visualization above, the tracing behavior can be recreated using the Sawyer robot model. Iterate through the joint configurations in jointConfigurationData to visualize the robot using show and continuously plot the end-effector position in 3-D space.

% For faster visualization, only display every few steps

vizStep = 5;

% Initialize a new figure window

figure

set(gcf,Visible="on");

title("Sawyer Tracing L-Membrane")

% Iterate through all joint configurations and end-effectort positions

for i = 1:vizStep:length(xPositionsEE)

show(sawyer, jointConfigurationData(:,i),Frames="off",PreservePlot=false);

hold on

plot3(xPositionsEE(1:i),yPositionsEE(1:i),zPositionsEE(1:i),"b",LineWidth=3)

view(135,20)

axis([-1 1 -.5 .5 -1 .75])

drawnow

end

hold off

rng(s); % restore the global state of the random number generator

complete code

clc; clear; close all;

%% ================== USER INPUT ==================

word = upper(input('Enter word to write: ', 's'));

numSamples = 20;

%% ================== LOAD ROBOT ==================

sawyer = importrobot('sawyer.urdf');

sawyer.DataFormat = 'column';

sawyer.Gravity = [0 0 -9.81];

eeName = 'right_hand';

%% ================== GENERATE WORD PATH ==================

[pathSegments, letterIdx] = generateWordPaths(word, numSamples);

%% ================== VISUALIZE TASK-SPACE PATH ==================

figure; hold on; grid on;

for i = 1:numel(pathSegments)

seg = pathSegments{i};

plot3(seg(:,1), seg(:,2), seg(:,3), 'r', 'LineWidth', 2);

end

xlabel('X'); ylabel('Y'); zlabel('Z');

title(['Task Space Path (XZ Plane) - "', word, '"']);

axis equal;

view(10,15);

hold off;

%% ================== SHOW ROBOT & PATH ==================

figure;

show(sawyer); hold on;

for i = 1:numel(pathSegments)

seg = pathSegments{i};

plot3(seg(:,1), seg(:,2), seg(:,3), 'k--', 'LineWidth', 1.5);

end

view(10,15);

axis([-1 1 -.6 .6 -0.8 0.8]);

hold off;

%% ================== INVERSE KINEMATICS ==================

ik = inverseKinematics('RigidBodyTree', sawyer);

ik.SolverParameters.AllowRandomRestarts = false;

weights = [1 1 1 0.2 0.2 0.2];

orientation = axang2rotm([0 1 0 pi]);

jointPath = [];

drawMask  = [];

%% ================== LETTER-BY-LETTER MOTION ==================

for L = 1:numel(letterIdx)

initialGuess = sawyer.homeConfiguration;

strokes = letterIdx{L};

firstSeg = pathSegments{strokes(1)};

pStart = firstSeg(1,:)';

% Travel move off plane (+Y)

Ttravel = [orientation [pStart(1); pStart(2)+0.1; pStart(3)]; 0 0 0 1];

qTravel = ik(eeName, Ttravel, weights, initialGuess);

jointPath(:,end+1) = qTravel;

drawMask(end+1) = false;

initialGuess = qTravel;

% Drawing strokes

for s = strokes

seg = pathSegments{s};

for k = 1:size(seg,1)

T = [orientation seg(k,:)'; 0 0 0 1];

q = ik(eeName, T, weights, initialGuess);

jointPath(:,end+1) = q;

drawMask(end+1) = true;

initialGuess = q;

end

end

end

%% ================== GUARANTEED UPWARD LIFT ==================

Tend = getTransform(sawyer, jointPath(:,end), eeName);

pEnd = tform2trvec(Tend)';

liftHeight = 0.25;

nLiftPts   = 25;

zLift = linspace(pEnd(3), pEnd(3)+liftHeight, nLiftPts);

qPrev = jointPath(:,end);

for i = 2:nLiftPts

pLift = [pEnd(1); pEnd(2); zLift(i)];

Tup = [orientation pLift; 0 0 0 1];

qUp = ik(eeName, Tup, weights, qPrev);

jointPath(:,end+1) = qUp;

drawMask(end+1) = false;

qPrev = qUp;

end

%% ================== END-EFFECTOR TRAJECTORY ==================

xEE = []; yEE = []; zEE = [];

for k = 1:size(jointPath,2)

T = getTransform(sawyer, jointPath(:,k), eeName);

p = tform2trvec(T);

if drawMask(k)

xEE(end+1) = p(1);

yEE(end+1) = p(2);

zEE(end+1) = p(3);

else

xEE(end+1) = NaN;

yEE(end+1) = NaN;

zEE(end+1) = NaN;

end

end

%% ================== ROBOT ANIMATION ==================

figure;

for k = 1:3:length(jointPath)

show(sawyer, jointPath(:,k), 'PreservePlot', false, 'Frames','off');

hold on

plot3(xEE(1:k), yEE(1:k), zEE(1:k), 'r', 'LineWidth', 2)

view(10,15)

axis([-1 1 -.6 .6 -0.8 0.8]);

drawnow

end

hold off;

%% ================== PATH GENERATOR (ALL LETTERS & NUMBERS) ==================

function [paths, letterIdx] = generateWordPaths(word, n)

yPlane  = -0.15;

scale   = 0.10;

spacing = 0.18;

xStart  = 0.45;

zBase   = 0.25;

paths = {};

letterIdx = {};

xOffset = 0;

L = @(x1,z1,x2,z2) ...

[linspace(x1,x2,n)', yPlane*ones(n,1), linspace(z1,z2,n)'];

for ch = word

switch ch

% ===== ALPHABETS =====

case 'A'

letter = {L(0,0,0.5,1), L(1,0,0.5,1), L(0.25,0.5,0.75,0.5)};

case 'B'

letter = {L(0,0,0,1), L(0,1,0.7,0.75), L(0.7,0.75,0,0.5), ...

L(0,0.5,0.7,0.25), L(0.7,0.25,0,0)};

case 'C'

letter = {L(1,1,0,1), L(0,1,0,0), L(0,0,1,0)};

case 'D'

letter = {L(0,0,0,1), L(0,1,1,0.5), L(1,0.5,0,0)};

case 'E'

letter = {L(0,0,0,1), L(0,1,1,1), L(0,0.5,0.8,0.5), L(0,0,1,0)};

case 'F'

letter = {L(0,0,0,1), L(0,1,1,1), L(0,0.5,0.8,0.5)};

case 'G'

letter = {L(1,1,0,1), L(0,1,0,0), L(0,0,1,0), L(1,0,1,0.5)};

case 'H'

letter = {L(0,0,0,1), L(1,0,1,1), L(0,0.5,1,0.5)};

case 'I'

letter = {L(0.5,0,0.5,1)};

case 'J'

letter = {L(1,1,1,0.2), L(1,0.2,0.5,0), L(0.5,0,0,0.2)};

case 'K'

letter = {L(0,0,0,1), L(0,0.5,1,1), L(0,0.5,1,0)};

case 'L'

letter = {L(0,1,0,0), L(0,0,1,0)};

case 'M'

letter = {L(0,0,0,1), L(0,1,0.5,0), L(0.5,0,1,1), L(1,1,1,0)};

case 'N'

letter = {L(0,0,0,1), L(0,1,1,0), L(1,0,1,1)};

case 'O'

letter = {L(0,0,1,0), L(1,0,1,1), L(1,1,0,1), L(0,1,0,0)};

case 'P'

letter = {L(0,0,0,1), L(0,1,1,1), L(1,1,1,0.5), L(1,0.5,0,0.5)};

case 'Q'

letter = {L(0,0,1,0), L(1,0,1,1), L(1,1,0,1), L(0,1,0,0), L(0.6,0.2,1,0)};

case 'R'

letter = {L(0,0,0,1), L(0,1,1,1), L(1,1,1,0.5), ...

L(1,0.5,0,0.5), L(0,0.5,1,0)};

case 'S'

letter = {L(1,1,0,1), L(0,1,0,0.5), L(0,0.5,1,0.5), ...

L(1,0.5,1,0), L(1,0,0,0)};

case 'T'

letter = {L(0,1,1,1), L(0.5,1,0.5,0)};

case 'U'

letter = {L(0,1,0,0), L(0,0,1,0), L(1,0,1,1)};

case 'V'

letter = {L(0,1,0.5,0), L(0.5,0,1,1)};

case 'W'

letter = {L(0,1,0.25,0), L(0.25,0,0.5,1), ...

L(0.5,1,0.75,0), L(0.75,0,1,1)};

case 'X'

letter = {L(0,1,1,0), L(0,0,1,1)};

case 'Y'

letter = {L(0,1,0.5,0.5), L(1,1,0.5,0.5), L(0.5,0.5,0.5,0)};

case 'Z'

letter = {L(0,1,1,1), L(1,1,0,0), L(0,0,1,0)};

% ===== NUMBERS =====

case '0'

letter = {L(0,0,1,0), L(1,0,1,1), L(1,1,0,1), L(0,1,0,0)};

case '1'

letter = {L(0.5,0,0.5,1)};

case '2'

letter = {L(0,1,1,1), L(1,1,1,0.5), L(1,0.5,0,0), L(0,0,1,0)};

case '3'

letter = {L(0,1,1,1), L(1,1,1,0), L(0,0.5,1,0.5), L(0,0,1,0)};

case '4'

letter = {L(0,1,0,0.5), L(0,0.5,1,0.5), L(1,1,1,0)};

case '5'

letter = {L(1,1,0,1), L(0,1,0,0.5), L(0,0.5,1,0.5), ...

L(1,0.5,1,0), L(1,0,0,0)};

case '6'

letter = {L(1,1,0,0.5), L(0,0.5,0,0), L(0,0,1,0), ...

L(1,0,1,0.5), L(1,0.5,0,0.5)};

case '7'

letter = {L(0,1,1,1), L(1,1,0.5,0)};

case '8'

letter = {L(0,0,1,0), L(1,0,1,1), L(1,1,0,1), ...

L(0,1,0,0), L(0,0.5,1,0.5)};

case '9'

letter = {L(1,0.5,1,1), L(1,1,0,1), L(0,1,0,0.5), L(0,0.5,1,0.5)};

otherwise

letter = {};

end

idx = [];

for k = 1:numel(letter)

seg = letter{k};

seg(:,1) = seg(:,1)*scale + xStart + xOffset;

seg(:,3) = seg(:,3)*scale + zBase;

paths{end+1} = seg;

idx(end+1) = numel(paths);

end

letterIdx{end+1} = idx;

xOffset = xOffset + spacing;

end

end

chatgpt link: https://chatgpt.com/share/6964c919-8dcc-8002-83d6-6b1e72af4f62